Series

Single Conductor Series Resistance Custom Cable Assembly for Snow Melting and De-Icing Applications

TXLP1

Features

Voltage	-	120V to 600V (max).

Cold lead	-	Standard length 15 ft. (4.57 m).

Longer lengths available (See Options table).

Cable diameter	-	6 mm to 6.5 mm (See Models table for details).

Bending radius, minimum	-	5x cable diameter.

Maximum operating

temperature	-	65 ⁰C (149 ⁰F).

Construction	-	Stranded resistance heating wire with XLPE insulation, tinned copper

grounding conductor, aluminum sheath, and PVC outer jacket.

Warranty	-	10-year limited warranty on the resistance cable.

Controls	-	The slab temperature must be monitored and controlled.

Requires a ground fault circuit-interrupter (GFCI).

Models

| Product # | Resistance (Ohms) Per metre | Cable Outer Diameter | Weight Per |  | Qty |
| --- | --- | --- | --- | --- | --- |
|  |  | mm | 100 m kg | 300 ft. lb |  |
| TXLP1-10156651 | 12.7 | 6.0 | 4.6 | 11.1 |  |
| TXLP1-10156650 | 7.7 | 6.0 | 4.6 | 11.1 |  |
| TXLP1-10156649 | 5.35 | 6.0 | 4.6 | 11.1 |  |
| TXLP1-10156648 | 3.5 | 6.1 | 4.9 | 11.8 |  |
| TXLP1-10156647 | 2.5 | 6.1 | 5.1 | 12.3 |  |
| TXLP1-10156646 | 1.4 | 6.1 | 5.0 | 12.0 |  |
| TXLP1-10156645 | 1.0 | 6.3 | 5.2 | 11.8 |  |
| TXLP1-10156644 | 0.7 | 6.3 | 5.1 | 12.3 |  |
| TXLP1-10156613 | 0.49 | 6.3 | 5.3 | 12.0 |  |
| TXLP1-10156612 | 0.3 | 6.3 | 5.3 | 12.8 |  |
| TXLP1-10156611 | 0.2 | 6.3 | 5.3 | 12.8 |  |
| TXLP1-10156610 | 0.13 | 6.5 | 5.6 | 12.5 |  |
| TXLP1-10156609 | 0.09 | 6.3 | 5.3 | 12.8 |  |
| TXLP1-10156608 | 0.07 | 6.5 | 5.6 | 13.4 |  |
| TXLP1-10156607 | 0.05 | 6.5 | 5.7 | 13.6 |  |
| TXLP1-10156606 | 0.02 | 6.5 | 5.8 | 13.8 |  |

Options

| Product # | Description | Qty |
| --- | --- | --- |
| Factory installed only |  |  |
| 25 | 25 ft. (2.3 m) cold lead |  |
| 50 | 50 ft. (15 m) cold lead |  |
| 75 | 75 ft. (23 m) cold lead |  |
| 100 | 100 ft. (30.48 m) cold lead |  |
| Kit |  |  |
| BRIPPS-75 | 75 ft. (23 m) galvanized steel cable clip strip for installation |  |
| KIT-SP3 | Repair kit |  |
| HT-2-SIGN | Lamacoid snow melt warning sign |  |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Electricial Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact at manufacturer: |  |
| Comments: |  |